# Implementing a Lockout/Tagout Program

Implementing a Lockout/Tagout (LOTO) program is essential in Career Technical Education (CTE) high-hazard programs to ensure the safety of students and staff. LOTO procedures are designed to prevent the unexpected release of hazardous energy during the servicing and maintenance of machinery and equipment. This process involves isolating energy sources and securing them in an "off" state to protect individuals from potential harm.

## Understanding Hazardous Energy

Hazardous energy refers to the various forms of energy that can pose risks if not properly controlled. In CTE environments, common types include:

- Electrical Energy: Energy from electrical sources, which can cause shocks or electrocution.

- Mechanical Energy: Energy from moving parts that can cause crushing or cutting injuries.

- Hydraulic and Pneumatic Energy: Pressurized fluids or gases that can lead to unexpected movement or release.

- Thermal Energy: High temperatures that can cause burns.

- Chemical Energy: Reactions that can release harmful substances or energy.

Controlling these energy sources is vital to prevent accidents during maintenance activities.

Key Components of a LOTO Program

- Energy Control Procedures: Develop clear, written site-specific procedures outlining the steps to isolate and control hazardous energy for each piece of equipment. These procedures should detail the specific steps for shutting down, isolating, blocking, and securing equipment to control hazardous energy. OSHA

- Employee Training: Provide comprehensive training for all students and staff on LOTO practices, ensuring they understand the importance of controlling hazardous energy and are competent in applying LOTO procedures. Training should cover the purpose and function of the energy control program, as well as the knowledge and skills required for the safe application, usage, and removal of energy controls. OSHA

- Periodic Inspections: Regularly inspect LOTO procedures to ensure compliance and effectiveness. This includes updating electrical panel labels, reviewing the procedures and correcting any deviations or inadequacies identified. OSHA

Implementing LOTO in CTE Programs

Incorporating LOTO procedures in CTE programs involves:

- Identifying Equipment: Determine which machines and equipment require LOTO during servicing or maintenance.

- Developing Procedures: Create specific LOTO procedures for each piece of equipment, detailing the steps to control hazardous energy.

- Training Students and Staff: Educate all individuals involved in operating or maintaining equipment on LOTO practices and the importance of controlling hazardous energy.

- Enforcing Compliance: Ensure adherence to LOTO procedures through supervision and regular audits.

## Conclusion

Implementing a Lockout/Tagout program in CTE high-hazard programs is crucial for preventing accidents and injuries related to the uncontrolled release of hazardous energy. By establishing clear procedures, providing thorough training, and ensuring compliance, educational institutions can create a safer environment that protects both students and staff.